# FOR IMMEDIATE RELEASE

# GREEN ANSWER FOR READYMIX RETURNS AT JUKSKEI QUARRY

AfriSam’s Jukskei Quarry in Midrand has cracked the problem of returned concrete with an environmentally-friendly solution that even pays its own way.

Every provider of readymix concrete must expect a certain amount of returned material as a result of unforeseen circumstances on the construction sites for which it is destined. Production superintendent at Jukskei Quarry, Mohamed Docrat, finally found a sustainable solution: to re-use the concrete in the company’s G5 sub-base product for road building.

Docrat and his management team focused on the idea of adding the recycled concrete to the quarry’s G5 product, as this was a product category that was permitted to contain material from multiple sources. By contrast, products within the G1 classification can only be from a single source.

“Also, the process of breaking up the returned concrete generates plenty of fines, which is beneficial to the product as the class specification for G5 demands that it must comprise about 80% fines,” he says.

A suitable process was devised and tested, in which an excavator-mounted hydraulic hammer breaks up the dry returns into 250 to 400 mm sizes, after which it is loaded by excavator into a dump truck and hauled to a secondary stockpile. The plant operators check that the material is within specification, and the blended material is then fed into a jaw crusher. The crushed material goes onto a production stockpile where it is homogenised, and it is taken by dump truck to a set-back stockpile where it is ready for sale.

Between 15% and 20% of recycled concrete is added into the G5, according to Docrat. “This gives us a good mixture of decomposed material and returned concrete,” he says. “During our G5 run to produce the material for sub-base layers in roads, we began testing by trickle-feeding a small portion of returned material into the G5, and found that it worked well.”

In line with AfriSam’s compliance with the South African Bureau of Standards’ COLTO (Committee of Land Transport Officials) material grading specifications, the G5 product containing the recycled concrete must pass all the relevant tests. The material is sampled first from the production stockpile, and then fully tested – from the set-back stockpile – for compliance with the final G5 specification.

According to Docrat, the final test of the idea was a trial on the Gauteng Freeway Improvement Project’s N1 extension, where the contractor tried the G5 product with recycled material and confirmed it was perfectly suitable.

This environmentally-friendly solution also means no slush around the Jukskei Quarry plant, less potential for contamination, and a smaller carbon footprint, he says; at the same time, it contributes to the recycling levels demanded by AfriSam’s stringent Environmental Management Programme.

PIC 01 : Mohamed Docrat, production superintendent at AfriSam’s Jukskei Quarry.

PIC 02 : The return concrete has also been used around the readymix plant to pave large areas.

PIC 03 : Waste at the facility that will be recycled.

ENDS … MAY 2017

FROM			:	CORALYNNE & ASSOCIATES

TEL : +27 011 849 3142

EMAIL : communicate@coralynne.co.za

WEBSITE: www.coralynne.co.za

FOR			:	MAXINE NEL

AFRISAM

TEL : +27 011 670 5893

EMAIL : maxine.nel@za.afrisam.com

www.afrisam.com